# WEB Quest 114 Networks

# Items highlighted in yellow are tasks for you to complete

Items in Blue italics  are help tips

You should find more links on the Internet  to help you answer questions, https://fcit.usf.edu/network/chap1/chap1.htm

this web quest includes most answers.

Questions are numbered and may be highlighted =  answer where you see this Answer

- What is a Network?

http://compnetworking.about.com/od/homenetworking/ig/Home-Network-Diagrams/

A network consists of two or more computers that are linked in order to share resources (such as printers and CDs), exchange files, or allow electronic communications. The computers on a network may be linked through cables, telephone lines, radio waves, satellites, or infrared light beams.

The two basic types of networks include:

- Local Area Network (LAN)

- Wide Area Network (WAN)

You may also see references to a Metropolitan Area Networks (MAN), a Wireless LAN (WLAN), or a Wireless WAN (WWAN).

1. What is a Local Area Network

http://searchnetworking.techtarget.com/definition/local-area-network-LAN

See the Topology, Cabling, and Hardware sections of this tutorial for more information on the configuration of a LAN.

2. What is Wide Area Network

http://searchenterprisewan.techtarget.com/definition/WAN

Wide Area Networks (WANs) connect

https://fcit.usf.edu/network/chap1/chap1.htm

3. What are the Advantages of Installing a School Network

4. What are the Disadvantages of Installing a School Network

5. What is a Protocol?

A protocol is a

The complexity of these layers is beyond the scope of this tutorial; however, they can be simplified into four layers to help identify some of the protocols with which you should be familiar (see fig 1).

| OSI Layer | Name |  | Common Protocols |  |
| --- | --- | --- | --- | --- |
| 7 | Application |  |  |  |
| 6 | Presentation |  | HTTP ¦ FTP ¦ SMTP ¦ DNS |  |
| 5 | Session |  |  |  |
| 4 | Transport |  | TCP | SPX |
| 3 | Network |  | IP | IPX |
| 2 | Data Link |  | Ethernet |  |
| 1 | Physical |  |  |  |

Fig 1. OSI model related to common network protocols

Figure 1 illustrates how some of the major protocols would correlate to the OSI model in order to communicate via the Internet. In this model, there are four layers, including:

- Ethernet (Physical/Data Link Layers)

- IP/IPX (Network Layer)

- TCP/SPX (Transport Layer)

- HTTP, FTP, Telnet, SMTP, and DNS (Session/Presentation/Application Layers)

Assuming you want to send an e-mail message to someone in Italy, we will examine the layers "from the bottom up" -- beginning with Ethernet (physical/data link kayers).

6. Explain Ethernet (Physical/Data Link Layers)

The physical layer of the network focuses on

### Ethernet

The original Ethernet standard was developed in 1983 and had a maximum speed of 10 Mbps (phenomonal at the time). The Ethernet protocol allows for bus, star, or tree topologies, depending on the type of cables used and other factors .

The current standard at the 10 Mbps level is 10BaseT. The "10" stands for the speed of transmission (10 megabits per second); the "Base" stands for "baseband" meaning it has full control of the wire on a single frequency; and the "T" stands for "twisted pair" cable. Older standards, such as 10Base2 and 10Base5, used coaxial cable, but these standards are seldom used in new installations. Fiber cable can also be used at this level in 10BaseFL.

### Fast Ethernet

The Fast Ethernet protocol supports transmission up to 100 Mbps. Fast Ethernet requires the use of different, more expensive network concentrators/hubs and network interface cards. In addition, category 5 twisted pair or fiber optic cable is necessary. Fast Ethernet standards include:

- 100BaseT - 100 Mbps over 2-pair category 5 or better UTP cable.

- 100BaseFX - 100 Mbps over fiber cable.

- 100BaseSX -100 Mbps over multimode fiber cable.

- 100BaseBX - 100 Mbps over single mode fiber cable.

### Gigabit Ethernet

Gigabit Ethernet standard is a protocol that has a transmission speed of 1 Gbps (1000 Mbps). It can be used with both fiber optic cabling and copper. The 1000BaseT, the copper cable used for Gigabit Ethernet (see the Cabling section for more information).

- 1000BaseT - 1000 Mbps over 2-pair category 5 or better UTP cable.

- 1000BaseTX - 1000 Mbps over 2-pair category 6 or better UTP cable.

- 1000BaseFX - 1000 Mbps over fiber cable.

- 1000BaseSX -1000 Mbps over multimode fiber cable.

- 1000BaseBX - 1000 Mbps over single mode fiber cable.

The Ethernet standards continue to evolve. with 10 Gigabit Ethernet (10,000 Mbps) and 100 Gigabit Ethernet (100,000 Mbps),

### Ethernet Protocol Summary

Protocol; Cable; Speed. Ethernet; Twisted Pair, Coaxial, Fiber; 10 Mbps. Fast Ethernet; Twisted Pair, Fiber; 100 Mbps. Gigabit Ethernet; Twisted Pair, Fiber; 1000 Mbps.

### LocalTalk

LocalTalk is a network protocol that was developed by Apple Computer, Inc. for Macintosh computers many years ago. LocalTalk adapters and special twisted pair cable can be used to connect a series of older computers through the serial port (current Macintosh computers have Ethernet cards and/or wireless adapters installed). A primary disadvantage of LocalTalk is speed. Its speed of transmission is only 230 Kbps.

7. Explain a Token Ring

The Token Ring protocol was developed by IBM in the mid-1980s. The access method used involves

8. What is IP and IPX (Network Layer)

The network layer is in charge of

9. Explain a TCP and SPX (Transport Layer)

The transport layer is concerned with efficient and reliable trsansportation),

10. What is HTTP, FTP, SMTP and DNS (Session/Presentation/Application Layers)

Several protocols overlap the session, presentation, and application layers of networks. There protocols listed below are a few of the more well-known:

- DNS - Domain Name System - translates network address (such as IP addresses) into terms understood by humans (such as URLs)

- DHCP - Dynamic Host Configuration Protocol - can automatically assign Internet addresses to computers and users

- FTP - File Transfer Protocol - a protocol that is used to transfer and manipulate files on the Internet

- HTTP - HyperText Transfer Protocol - An Internet-based protocol for sending and receiving webpages

- IMAP - Internet Message Access Protocol - A protocol for e-mail messages on the Internet

- IRC - Internet Relay Chat - a protocol used for Internet chat and other communications

- POP3 - Post Office protocol Version 3 - a protocol used by e-mail clients to retrieve messages from remote servers

- SMTP - Simple Mail Transfer Protocol - A protocol for e-mail messages on the Internet

11.Describe Networking Hardware?

Networking hardware includes all

This section provides information on the following components:

- File Servers

- Workstations

- Network Interface Cards

- Switches

- Repeaters

- Bridges

- Routers

12. Explain File Servers

A file server stands at the heart of most networks.

- 800 megahertz or faster microprocessor (Pentium 3 or 4, G4 or G5)

- A fast hard drive with at least 120 gigabytes of storage

- A RAID (Redundant Array of Inexpensive Disks) to preserve data after a disk casualty

- A tape back-up unit (i.e. DAT, JAZ, Zip, or CD-RW drive)

- Numerous expansion slots

- Fast network interface card

- At least of 512 MB of RAM

13. What is a Workstations

All of the user computers connected to a network are called workstations. A typical workstation is a

14. What are Network Interface Cards

The network interface card (NIC) provides the

### When would you use an Ethernet Card

Ethernet cards are usually purchased separately from a computer, although many computers (such as the Macintosh) now include an option for a pre-installed Ethernet card. Ethernet cards contain connections for either coaxial or twisted pair cables (or both) (See fig. 1). If it is designed for coaxial cable, the connection will be BNC. If it is designed for twisted pair, it will have a RJ-45 connection. Some Ethernet cards also contain an AUI connector. This can be used to attach coaxial, twisted pair, or fiber optics cable to an Ethernet card. When this method is used there is always an external transceiver attached to the workstation. (See the Cabling section for more information on connectors.)

Fig. 1. Ethernet card.
From top to bottom:
RJ-45, AUI, and BNC connectors

### LocalTalk Connectors

LocalTalk is Apple's built-in solution for networking older Macintosh computers. It utilized a special adapter box and a cable that plugged into the printer port of a Macintosh. A major disadvantage of LocalTalk was that it is slow (only 230 Kbps) in comparison to Ethernet; therefore, it is rarely used in current networks.

### Token Ring Cards

Token Ring network cards look similar to Ethernet cards and were popular in IBM computers. They are seldom used with current networks.

## Switches

A concentrator is a device that provides a central connection point for cables from workstations, servers, and peripherals. In a star topology, twisted-pair wire is run from each workstation to a central switch/hub. Most switches are active, that is they electrically amplify the signal as it moves from one device to another. Switches no longer broadcast network packets as hubs did in the past, they memorize addressing of computers and send the information to the correct location directly. Switches are:

- Usually configured with 8, 12, or 24 RJ-45 ports

- Often used in a star or tree topology

- Sold with specialized software for port management

- Also called hubs

- Usually installed in a standardized metal rack that also may store netmodems, bridges, or routers

## Repeaters

Since a signal loses strength as it passes along a cable, it is often necessary to boost the signal with a device called a repeater. The repeater electrically amplifies the signal it receives and rebroadcasts it. Repeaters can be separate devices or they can be incorporated into a concentrator. They are used when the total length of your network cable exceeds the standards set for the type of cable being used.

A good example of the use of repeaters would be in a local area network using a star topology with unshielded twisted-pair cabling. The length limit for unshielded twisted-pair cable is 100 meters. The most common configuration is for each workstation to be connected by twisted-pair cable to a multi-port active concentrator. The concentrator amplifies all the signals that pass through it allowing for the total length of cable on the network to exceed the 100 meter limit.

## Bridges

A bridge is a device that allows you to segment a large network into two smaller, more efficient networks. If you are adding to an older wiring scheme and want the new network to be up-to-date, a bridge can connect the two.

A bridge monitors the information traffic on both sides of the network so that it can pass packets of information to the correct location. Most bridges can "listen" to the network and automatically figure out the address of each computer on both sides of the bridge. The bridge can inspect each message and, if necessary, broadcast it on the other side of the network.

The bridge manages the traffic to maintain optimum performance on both sides of the network. You might say that the bridge is like a traffic cop at a busy intersection during rush hour. It keeps information flowing on both sides of the network, but it does not allow unnecessary traffic through. Bridges can be used to connect different types of cabling, or physical topologies. They must, however, be used between networks with the same protocol.

15. Explain the use of Routers

A

Routers can:

- Direct signal traffic efficiently

- Route messages between any two protocols

- Route messages between linear bus, star, and star-wired ring topologies

- Route messages across fiber optic, coaxial, and twisted-pair cabling

16. What is Network Cabling?

Cable is the medium through which The following sections discuss the types of cables used in networks and other related topics.

- Unshielded Twisted Pair (UTP) Cable

- Shielded Twisted Pair (STP) Cable

- Coaxial Cable

- Fiber Optic Cable

- Cable Installation Guides

- Wireless LANs

17. Unshielded Twisted Pair (UTP) Cable

Twisted pair cabling

### Categories of Unshielded Twisted Pair

Category; Speed; Use. 1; 1 Mbps; Voice Only (Telephone Wire). 2; 4 Mbps; LocalTalk & Telephone (Rarely used). 3; 16 Mbps; 10BaseT Ethernet. 4; 20 Mbps; Token Ring (Rarely used). 5; 100 Mbps (2 pair) 1000 Mbps (4 pair); 100BaseT Ethernet Gigabit Ethernet. 5e; 1,000 Mbps; Gigabit Ethernet. 6; 10,000 Mbps; Gigabit Ethernet.

18. Unshielded Twisted Pair Connector

The standard connector

19. Shielded Twisted Pair (STP) Cable

Although UTP cable is the least expensive cable, it may be susceptible to radio and electrical frequency interference (it should not be too close to electric motors, fluorescent lights, etc.).

- Each pair of wires is individually shielded with foil.

- There is a foil or braid shield inside the jacket covering all wires (as a group).

- There is a shield around each individual pair, as well as around the entire group of wires (referred to as double shield twisted pair).

20. Coaxial Cable

Coaxial cabling has

The two types of coaxial

Thin coaxial cable is also referred to as thinnet. 10Base2 refers to the specifications for thin coaxial cable carrying Ethernet signals. The 2 refers to the approximate maximum segment length being 200 meters. In actual fact the maximum segment length is 185 meters. Thin coaxial cable has been popular in

Thick coaxial cable is also referred to as thicknet. 10Base5 refers to the specifications for thick coaxial cable carrying Ethernet signals. The 5 refers to the maximum segment length being 500 meters. Thick coaxial cable has an extra protective plastic cover that helps keep moisture away from the center conductor. This makes thick coaxial a great choice when running longer lengths in a linear bus network. One disadvantage of thick coaxial is that it does

21. Coaxial Cable Connectors

The most common type of connector used with coaxial cables is the BNC

22. Fiber Optic Cable

Fiber optic cabling consists of

There are two common types of fiber cables -- single mode and multimode. Multimode cable has a larger diameter; however, both cables provide high bandwidth at high speeds. Single mode can provide more distance, but it is more expensive.

### Ethernet Cable Summary

Specification; Cable Type. 10BaseT; Unshielded Twisted Pair. 10Base2; Thin Coaxial. 10Base5; Thick Coaxial. 100BaseT; Unshielded Twisted Pair. 100BaseFX; Fiber Optic. 100BaseBX; Single mode Fiber. 100BaseSX; Multimode Fiber. 1000BaseT; Unshielded Twisted Pair. 1000BaseFX; Fiber Optic. 1000BaseBX; Single mode Fiber. 1000BaseSX; Multimode Fiber.

## Installing Cable - Some Guidelines

When running cable, it is best to follow a few simple rules:

- Always use more cable than you need. Leave plenty of slack.

- Test every part of a network as you install it. Even if it is brand new, it may have problems that will be difficult to isolate later.

- Stay at least 3 feet away from fluorescent light boxes and other sources of electrical interference.

- If it is necessary to run cable across the floor, cover the cable with cable protectors.

- Label both ends of each cable.

- Use cable ties (not tape) to keep cables in the same location together.

## Wireless LANs

More and more networks are operating without cables, in the wireless mode. Wireless LANs use high frequency radio signals, infrared light beams, or lasers to communicate between the workstations and the file server or hubs. Each workstation and file server on a wireless network has some sort of transceiver/antenna to send and receive the data. Information is relayed between transceivers as if they were physically connected. For longer distance, wireless communications can also take place through cellular telephone technology, microwave transmission, or by satellite.

Wireless networks are great for allowing laptop computers or remote computers to connect to the LAN. Wireless networks are also beneficial in older buildings where it may be difficult or impossible to install cables.

The two most common types of infrared communications used in schools are line-of-sight and scattered broadcast. Line-of-sight communication means that there must be an unblocked direct line between the workstation and the transceiver. If a person walks within the line-of-sight while there is a transmission, the information would need to be sent again. This kind of obstruction can slow down the wireless network. Scattered infrared communication is a broadcast of infrared transmissions sent out in multiple directions that bounces off walls and ceilings until it eventually hits the receiver. Networking communications with laser are virtually the same as line-of-sight infrared networks.

### Wireless standards and speeds

The Wi-Fi Alliance is a global, non-profit organization that helps to ensure standards and interoperability for wireless networks, and wireless networks are often referred to as WiFi (Wireless Fidelity). The original Wi-Fi standard (IEEE 802.11) was adopted in 1997. Since then many variations have emerged (and will continue to emerge). Wi-Fi networks use the Ethernet protocol.

| Standard | Max Speed | Typical Range |
| --- | --- | --- |
| 802.11a | 54 Mbps | 150 feet |
| 802.11b | 11 Mbps | 300 feet |
| 802.11g | 54 Mbps | 300 feet |

23. What are the Advantages of wireless networks:

24. What are the Disadvantages of wireless networks:

26. What is a Topology?

## http://compnetworking.about.com/od/networkdesign/ig/Computer-Network-Topologies/

The physical topology of a network refers to the configuration of cables, computers, and other peripherals. Physical topology should not be confused with logical topology which is the method used to pass information between workstations. Logical topology was discussed in the Protocol chapter .

### Main Types of Physical Topologies

The following sections discuss the physical topologies used in networks and other related topics.

- Linear Bus

- Star

- Tree (Expanded Star)

- Considerations When Choosing a Topology

- Summary Chart

## Linear Bus

A linear bus topology consists of a main run of cable with a terminator at each end (See fig. 1). All nodes (file server, workstations, and peripherals) are connected to the linear cable.

Fig. 1. Linear Bus topology

### Advantages of a Linear Bus Topology

- Easy to connect a computer or peripheral to a linear bus.

- Requires less cable length than a star topology.

### Disadvantages of a Linear Bus Topology

- Entire network shuts down if there is a break in the main cable.

- Terminators are required at both ends of the backbone cable.

- Difficult to identify the problem if the entire network shuts down.

- Not meant to be used as a stand-alone solution in a large building.

## Star

A star topology is designed with each node (file server, workstations, and peripherals) connected directly to a central network hub, switch, or concentrator (See fig. 2).

Data on a star network passes through the hub, switch, or concentrator before continuing to its destination. The hub, switch, or concentrator manages and controls all functions of the network. It also acts as a repeater for the data flow. This configuration is common with twisted pair cable; however, it can also be used with coaxial cable or fiber optic cable.

Fig. 2. Star topology

27. Advantages of a Star Topology

28. Disadvantages of a Star Topology

## Tree or Expanded Star

A tree topology combines characteristics of linear bus and star topologies. It consists of groups of star-configured workstations connected to a linear bus backbone cable (See fig. 3). Tree topologies allow for the expansion of an existing network, and enable schools to configure a network to meet their needs.

Fig. 3. Tree topology

### Advantages of a Tree Topology

- Point-to-point wiring for individual segments.

- Supported by several hardware and software venders.

### Disadvantages of a Tree Topology

- Overall length of each segment is limited by the type of cabling used.

- If the backbone line breaks, the entire segment goes down.

- More difficult to configure and wire than other topologies.

### 5-4-3 Rule

A consideration in setting up a tree topology using Ethernet protocol is the 5-4-3 rule. One aspect of the Ethernet protocol requires that a signal sent out on the network cable reach every part of the network within a specified length of time. Each concentrator or repeater that a signal goes through adds a small amount of time. This leads to the rule that between any two nodes on the network there can only be a maximum of 5 segments, connected through 4 repeaters/concentrators. In addition, only 3 of the segments may be populated (trunk) segments if they are made of coaxial cable. A populated segment is one that has one or more nodes attached to it . In Figure 4, the 5-4-3 rule is adhered to. The furthest two nodes on the network have 4 segments and 3 repeaters/concentrators between them.

This rule does not apply to other network protocols or Ethernet networks where all fiber optic cabling or a combination of a fiber backbone with UTP cabling is used. If there is a combination of fiber optic backbone and UTP cabling, the rule is simply translated to a 7-6-5 rule.

29. Considerations When Choosing a Topology

### Summary Chart

Physical Topology; Common Cable; Common Protocol. Linear Bus; Twisted Pair Coaxial Fiber; Ethernet. Star; Twisted Pair Fiber; Ethernet. Tree; Twisted Pair Coaxial Fiber; Ethernet.

30. What is a Network Operating System?

Unlike operating systems, such as Windows,

The two major types of network operating systems are:

- Peer-to-Peer

- Client/Server

31. Explain Peer-to-Peer

Peer-to-peer network operating systems allow

Fig. 1. Peer-to-peer network

33. Advantages of a peer-to-peer network:

34. Disadvantages of a peer-to-peer network:

## Client/Server

Client/server network operating systems allow the network to centralize functions and applications in one or more dedicated file servers (See fig. 2). The file servers become the heart of the system, providing access to resources and providing security. Individual workstations (clients) have access to the resources available on the file servers. The network operating system provides the mechanism to integrate all the components of the network and allow multiple users to simultaneously share the same resources irrespective of physical location. Novell Netware and Windows 2000 Server are examples of client/server network operating systems.

Fig. 2. Client/server network

35. Advantages of a client/server network:

36. Disadvantages of a client/server network:

## Network Operating System Software

The following links include some of the more popular peer-to-peer and client/server network operating systems.

- Macintosh OSX

- Microsoft Windows Server

- Novell

- UNIX

GLOSSARY

- 10Base2 - Ethernet specification for thin coaxial cable, transmits signals at 10 Mbps (megabits per second) with a distance limit of 185 meters per segment.

- 10Base5 - Ethernet specification for thick coaxial cable, transmits signals at 10 Mbps (megabits per second) with a distance limit of 500 meters per segment.

- 10BaseF - Ethernet specification for fiber optic cable, transmits signals at 10 Mbps (megabits per second) with a distance limit of 2000 meters per segment.

- 10BaseT - Ethernet specification for unshielded twisted pair cable (category 3, 4, or 5), transmits signals at 10 Mbps (megabits per second) with a distance limit of 100 meters per segment.

- 100BaseT - Ethernet specification for unshielded twisted pair cabling that is used to transmit data at 100 Mbps (megabits per second) with a distance limit of 100 meters per segment.

- 1000BaseTX -Ethernet specification for unshielded twisted pair cabling that is used to trasmit data at 1 Gbps (gigabits per second) with a distance limitation of 220 meters per segment.

- Asynchronous Transfer Mode (ATM) - A network protocol that transmits data at a speed of 155 Mbps and higher. It is most often used to interconnect two or more local area networks.

- AppleTalk - Apple Computer's network protocol originally designed to run over LocalTalk networks, but can also run on Ethernet and Token Ring.

- AUI Connector (Attachment Unit Interface) - A 15 pin connector found on Ethernet cards that can be used for attaching coaxial, fiber optic, or twisted pair cable.

- Backbone - A cable to which multiple nodes or workstations are attached.

- Bit - Binary digit in the binary numbering system. Its value can be 0 or 1. In an 8-bit character scheme, it takes 8 bits to make a byte (character) of data.

- BNC Connector (Bayone-Neill-Concelman) - Standard connector used to connect 10Base2 coaxial cable.

- Bridge - Devices that connect and pass packets between two network segments that use the same communications protocol.

- Cable - Transmission medium of copper wire or optical fiber wrapped in a protective cover.

- Client/Server - A networking system in which one or more file servers (Server) provide services; such as network management, application and centralized data storage for workstations (Clients).

- CSMA/CA - Carrier Sense Multiple Access Collision Avoidance is a network access method in which each device signals its intent to transmit before it actually does so. This prevents other devices from sending information, thus preventing collisions from occurring between signals from two or more devices. This is the access method used by LocalTalk.

- CSMA/CD - Carrier Sense Multiple Access Collision Detection is a network access method in which devices that are ready to transmit data first check the channel for a carrier. If no carrier is sensed, a device can transmit. If two devices transmit at once, a collision occurs and each computer backs off and waits a random amount of time before attempting to retransmit. This is the access method used by Ethernet.

- Coaxial Cable - Cable consisting of a single copper conductor in the center surrounded by a plastic layer for insulation and a braided metal outer shield.

- Concentrator - A device that provides a central connection point for cables from workstations, servers, and peripherals. Most concentrators contain the ability to amplify the electrical signal they receive.

- DIN - A plug and socket connector consisting of a circular pattern of pins in a metal sleeve. This type of connector is commonly seen on keyboards.

- Dumb Terminal - Refers to devices that are designed to communicate exclusively with a host (main frame) computer. It receives all screen layouts from the host computer and sends all keyboard entry to the host. It cannot function without the host computer.

- E-mail - An electronic mail message sent from a host computer to a remote computer.

- End User - Refers to the human executing applications on the workstation.

- Ethernet - A network protocol invented by Xerox Corporation and developed jointly by Xerox, Intel and Digital Equipment Corporation. Ethernet networks use CSMA/CD and run over a variety of cable types at 10 Mbps (megabits per second).

- Expansion Slot - Area in a computer that accepts additional input/output boards to increase the capability of the computer.

- Fast Ethernet - A new Ethernet standard that supports 100 Mbps using category 5 twisted pair or fiber optic cable.

- Fiber Distributed Data Interface (FDDI) - A network protocol that is used primarily to interconnect two or more local area networks, often over large distances.

- Fiber Optic Cable - A cable, consisting of a center glass core surrounded by layers of plastic, that transmits data using light rather than electricity. It has the ability to carry more information over much longer distances.

- File Server - A computer connected to the network that contains primary files/applications and shares them as requested with the other computers on the network. If the file server is dedicated for that purpose only, it is connected to a client/server network. An example of a client/server network is Novell Netware. All the computers connected to a peer-to-peer network are capable of being the file server. Two examples of peer-to-peer networks are LANtastic and Windows for Workgroups.

- Gigabit Ethernet - An Ethernet protocol that raises the transmission rates to 1 Gbps (gigabits per second). It is primarily used for a high speed backbone of a network.

- Gigabyte (GB) - One billion bytes of information. One thousand megabytes.

- Hub - A hardware device that contains multiple independent but connected modules of network and internetwork equipment. Hubs can be active (where they repeat signals sent through them) or passive (where they do not repeat but merely split signals sent through them).

- Infrared - Electromagnetic waves whose frequency range is above that of microwaves, but below that of the visible spectrum.

- Intranet - Network internal to an organization that uses Internet protocols.

- Internet - A global network of networks used to exchange information using the TCP/IP protocol. It allows for electronic mail and the accessing ad retrieval of information from remote sources.

- LAN (Local Area Network) - A network connecting computers in a relatively small area such as a building.

- Linear Bus - A network topology in which each node attaches directly to a common cable.

- LocalTalk - Apple Corporation proprietary protocol that uses CSMA/CA media access scheme and supports transmissions at speeds of 230 Kbps (Kilobits per second).

- MAN (Metropolitan Area Network) - A network connecting computers over a large geographical area, such as a city or school district.

- MAU (Multistation Access Unit) - A Token Ring wiring hub.

- Modem (Modulator/Demodulator) - Devices that convert digital and analog signals. Modems allow computer data (digital) to be transmitted over voice-grade telephone lines (analog).

- Multiplexer - A device that allows multiple logical signals to be transmitted simultaneously across a single physical channel.

- Network Modem - A modem connected to a Local Area Network (LAN) that is accessible from any workstation on the network.

- Network Interface Card (NIC) - A board that provides network communication capabilities to and from a computer.

- Network Operating System (NOS) - Operating system designed to pass information and communicate between more than one computer. Examples include AppleShare, Novell NetWare, and Windows NT Server.

- Node - End point of a network connection. Nodes include any device attached to a network such as file servers, printers, or workstations.

- Node Devices - Any computer or peripheral that is connected to the network.

- PCMCIA - An expansion slot found in many laptop computers.

- Peer-to-Peer Network - A network in which resources and files are shared without a centralized management source.

- Physical Topology - The physical layout of the network; how the cables are arranged; and how the computers are connected.

- Point-to-Point - A direct link between two objects in a network.

- Ports - A connection point for a cable.

- Protocol -A formal description of a set of rules and conventions that govern how devices on a network exchange information.

- RAID (Redundant Array of Inexpensive Disks) - A configuration of multiple disks designed to preserve data after a disk casualty.

- RAM (Random Access Memory) - The working memory of a computer where data and programs are temporarily stored. RAM only holds information when the computer is on.

- Repeater - A device used in a network to strengthen a signal as it is passed along the network cable.

- RJ-45 - Standard connectors used for unshielded twisted-pair cable.

- Router -A device that routes information between interconnected networks. It can select the best path to route a message, as well as translate information from one network to another. It is similar to a superintelligent bridge.

- SCSI (Small Computer Serial Interface) - An interface controller that allows several peripherals to be connected to the same port on a computer.

- Segment - Refers to a section of cable on a network. In Ethernet networks, two types of segments are defined. A populated or trunk segment is a network cable that has one or more nodes attached to it. A link segment is a cable that connects a computer to an interconnecting device, such as a repeater or concentrator, or connects a interconnecting device to another interconnecting device.

- Sneaker-Net - Refers to a manual method of sharing files in which a file is copied from a computer to a floppy disk, transported to a second computer by a person physically walking (apparently wearing sneakers) to the second computer, and manually transferring the file from floppy disk to the second computer.

- Speed of Data Transfer - The rate at which information travels through a network, usually measured in megabits per second.

- Star Topology - LAN topology in which each node on a network is connected directly to a central network hub or concentrator.

- Star-Wired Ring - Network topology that connects network devices (such as computers and printers) in a complete circle.

- Switch - A "intelligent" type of hub, in that it sends packets only to the intended ports, rather than all computers on the network.

- Tape Back-Up - Copying all the data and programs of a computer system on magnetic tape. On tape, data is stored sequentially. When retrieving data, the tape is searched from the beginning of tape until the data is found.

- Terminator - A device that provides electrical resistance at the end of a transmission line. Its function is to absorb signals on the line, thereby keeping them from bouncing back and being received again by the network.

- Thicknet - A thick coaxial cable that is used with a 10Base5 Ethernet LAN.

- Thinnet - A thin coaxial cable that is used with a 10Base2 Ethernet LAN.

- Token - A special packet that contains data and acts as a messenger or carrier between each computer and device on a ring topology. Each computer must wait for the messenger to stop at its node before it can send data over the network.

- Token Ring - A network protocol developed by IBM in which computers access the network through token-passing. Usually uses a star-wired ring topology.

- Topology - There are two types of topology: physical and logical. The physical topology of a network refers to the configuration of cables, computers, and other peripherals. Logical topology is the method used to pass the information between workstations. Issues involving logical topologies are discussed on the Protocol chapter

- Transceiver (Transmitter/Receiver) - A Device that receives and sends signals over a medium. In networks, it is generally used to allow for the connection between two different types of cable connectors, such as AUI and RJ-45.

- Tree Topology - LAN topology similar to linear bus topology, except that tree networks can contain branches with multiple nodes.

- Twisted Pair - Network cabling that consists of four pairs of wires that are manufactured with the wires twisted to certain specifications. Available in shielded and unshielded versions.

- USB (Universal Serial Bus) Port - A hardware interface for low-speed peripherals such as the keyboard, mouse, joystick, scanner, printer, and telephony devices.

- WAN (Wide Area Network) - A network connecting computers within very large areas, such as states, countries, and the world.

- Workgroup - A collection of workstations and servers on a LAN that are designated to communicate and exchange data with one another.

- Workstation - A computer connected to a network at which users interact with software stored on the network.