WEB Quest 08  How Computers Work

Items in blue offer some information and notes and items underlined are links to help you get answers. Questions are numbered and may be highlighted =  answer where you see this Answer:

Task Brief:   How  Computer Components Work

Computers work thanks to a series of hardware devices that are closely interconnected. When looking at a computer from the outside, the basic parts are the system case, which may be a tower for a desktop machine or the body of a laptop;

- What is a computer?

http://www.techterms.com/

Answer:

A mouse or trackpad may or may not be essential to the functioning of a computer, depending on the type and model.

2. How does a mouse differ from a  trackpad?

Answer:

The motherboard is central point of the computer, where all the various components attach and communicate with each other. Key to allowing a computer to work is the central processing unit (CPU), the central stop for all the processes the computer goes through. As a command is sent, such as "open a program" or "turn the monitor on," the CPU interprets this order and then acts accordingly.

3. What is the central part of a computer?

Answer:

Once the computer is turned on, or booted up, the CPU goes on to activate certain sections so that it can then give you access to programs and processes. Computers work based on the CPU granting access to users, so if the booting up process malfunctions, it can mean that the computer cannot be used, even if everything else inside is working properly.

Memory is also extremely important to allow a computer to work. The two main kinds of memory are Random Access Memory (RAM) and Read-Only Memory (ROM). ROM is stored data, and cannot be written to; RAM is memory that can be read from and written to, allowing new data to be saved. In many cases, additional RAM can be added.

Random access memory (RAM) is the best known form of computer memory. RAM is considered "random access" because you can access any memory cell directly if you know the row and column that intersect at that cell.

The opposite of RAM is serial access memory (SAM). SAM stores data as a series of memory cells that can only be accessed sequentially (like a cassette tape). If the data is not in the current location, each memory cell is checked until the needed data is found. SAM works very well for memory buffers, where the data is normally stored in the order in which it will be used (a good example is the texture buffer memory on a video card). RAM data, on the other hand, can be accessed in any order.

Similar to a microprocessor, a memory chip is an integrated circuit (IC) made of millions of transistors and capacitors. In the most common form of computer memory, dynamic random access memory (DRAM), a transistor and a capacitor are paired to create a memory cell, which represents a single bit of data. The capacitor holds the bit of information -- a 0 or a 1 (see How Bits and Bytes Work for information on bits). The transistor acts as a switch that lets the control circuitry on the memory chip read the capacitor or change its state.

4. More on RAM (Print screen your results here) RAM Quiz Does adding RAM make your computer faster? Answer: PlanetGreen.com: Computer Tune Up

5. Give examples of RAM Memory chips (size type etc.)

Answer:

One kind of ROM is the Basic Input/Output System (BIOS), which is in charge of communications between the software on a computer and the hardware. When a computer is first turned on, the BIOS checks basic data such as hard memory, RAM, any cards installed, and other devices. BIOS also checks for booting up errors and offers to fix them if necessary.

Other less central — but no less vital — parts that let computers work include the power supply, transformer, and battery. These parts make sure each component gets the electricity it needs in the proper amount, and that key information is saved even when the power is off. The computer drives, including hard drives, flash drives, and any drives with removable media, such as CD-ROM drives, allow the user to upload new data and applications to the computer and save files. The cooling system helps keep all of the components from overheating.

Most computers also have other components without which a computer would be more difficult to use. Graphics cards allow the computer to display graphics on the monitor, and come in many different levels. Sounds cards allow the computer to play sounds. Connecting to the Internet or other computers requires a modem. Most computers come with all of these components, and often it is possible for the owner to upgrade each to newer or more advanced versions.

Input/output (I/O) is the name given to the processes or components needed to interact with the CPU and make computers work. These include the monitor and keyboard, but also CD-ROMs and removable flash memory cards. Input/output processes allow you to order the computer to do something, making the essential for interaction and use.

6.  List some memory devices (size, type)

Answer:

The Computer Brain

When you first turn on a typical computer, it must run an initial program that sets up the operating system. This initial program has to run even before the computer is able to interact with its hard drive, so the program must be available at the very instant the computer's power becomes available. Read-only memory is used for this initial bootup operation. Unlike normal random access memory, which is usually "volatile" and loses its stored information when power is removed, read-only memory retains its information without power. When you turn on the computer, this read-only memory provides the instructions the computer uses to begin loading the operating system from the hard drive. A computer chip is also known as a digital integrated circuit. It is typically a thin wafer of silicon, cut from a single crystal of that element. The surface of the wafer has been chemically modified and it has had intricate patterns of aluminum wires and other structures cut and deposited photographically on its surface to form enormous numbers of transistors and other special structures. Each of these transistors is an electronically controllable switch. A tiny adjustment in the electric charge on the control element of one of these transistors—its gate—can dramatically alter that transistor's current carrying ability. These transistors work together to perform task that range from remembering one bit of information to multiplying two huge numbers together. The millions of transistors on a typical computer chip are able to perform extremely complicated tasks, as we see everyday in modern computers. A digital computer's hardware is a complex system of four functionally different elements—a central processing unit, input devices, memory-storage devices, and output devices—linked by a communication network, or bus. The bus is usually incorporated into the main circuit board, called the motherboard, which is plugged into all the other components. 7. Different Computers (Mac, Windows etc.) Run from different Chips name at least two types. Answers: 1 2 Bits A computer chip is also known as a digital integrated circuit. It is typically a thin wafer of silicon, cut from a single crystal of that element. The surface of the wafer has been chemically modified and it has had intricate patterns of aluminum wires and other structures cut and deposited photographically on its surface to form enormous numbers of transistors and other special structures. Each of these transistors is an electronically controllable switch. A tiny adjustment in the electric charge on the control element of one of these transistors—its gate—can dramatically alter that transistor's current carrying ability. These transistors work together to perform task that range from remembering one bit of information to multiplying two huge numbers together. The millions of transistors on a typical computer chip are able to perform extremely complicated tasks, as we see everyday in modern computers. bit: The word "bit" is short for "binary digit." A bit is the smallest piece of computer information. byte: Most computers use combinations of eight bits, called bytes, to represent one character of data. For example, the word "cat" has three characters, and it would be represented by three bytes. kilobyte (K or KB): A kilobyte is equal to 1,024 bytes. megabyte (MB) A megabyte is equal to 1,048,576 bytes, but it is usually rounded off to one million bytes. gigabyte (GB): A gigabyte is one thousand megabytes. terabyte (TB): A terabyte is one trillion bytes, or 1000 gigabytes. Computer memory is usually measured in megabytes or gigabytes. This tells how much information your computer can store. 8. How Big Is a Bit? Answers: The speed of a modem (a device that connects two computers over a telephone line) is measured in bits per second, or bps. This tells how much information can be sent in a second. A computer generally works with groups of bits at a time. A group of eight bits is called a byte. A byte can represent the 256 different binary values 00000000 through 11111111, which are equal to the decimal values 0 through 255. That is enough values to assign a numeric code to each letter of the Latin alphabet (both upper and lower case, plus some accented letters), the 10 decimal digits, punctuation marks, and common mathematical and other special symbols. Therefore, depending on a program's context, the binary value 01000001 can represent the decimal value 65, the capital letter A, or an instruction to the computer to move data from one place to another. The amount of data that can be stored in a computer's memory or on a disk is referred to in terms of numbers of bytes. Computers can store billions of bytes in their memory, and a modern disk can hold tens, or even hundreds, of billions of bytes of data. To deal with such large numbers, the abbreviations K, M, and G (for “kilo,” “mega,” and “giga,” respectively) are often used. K stands for 210 (1,024, or about a thousand), M stands for 220 (1,048,576, or about a million), and G stands for 230 (1,073,741,824, or about a billion). The abbreviation B stands for byte, and b for bit. So a computer that has a 256 MB (megabyte) memory can hold about 256 million characters. An 80 GB (gigabyte) disk stores about 80 billion characters. A capacitor is like a small bucket that is able to store electrons. To store a 1 in the memory cell, the bucket is filled with electrons. To store a 0, it is emptied. The problem with the capacitor's bucket is that it has a leak. In a matter of a few milliseconds a full bucket becomes empty. Therefore, for dynamic memory to work, either the CPU or the memory controller has to come along and recharge all of the capacitors holding a 1 before they discharge. To do this, the memory controller reads the memory and then writes it right back. This refresh operation happens automatically thousands of times per second. http://computer.howstuffworks.com/inside-computer3.htm The capacitor in a dynamic RAM memory cell is like a leaky bucket. It needs to be refreshed periodically or it will discharge to 0. This refresh operation is where dynamic RAM gets its name. Dynamic RAM has to be dynamically refreshed all of the time or it forgets what it is holding. The downside of all of this refreshing is that it takes time and slows down the memory. In this article, you'll learn all about what RAM is, what kind you should buy and how to install it. See the next page to learn more about dynamic RAM and memory cells. Your Browser Does Not Support iFrames Memory Cells and DRAM Memory is made up of bits arranged in a two-dimensional grid. In this figure, red cells represent 1s and white cells represent 0s. In the animation, a column is selected and then rows are charged to write data into the specific column. Memory cells are etched onto a silicon wafer in an array of columns (bitlines) and rows (wordlines). The intersection of a bitline and wordline constitutes the address of the memory cell. DRAM works by sending a charge through the appropriate column (CAS) to activate the transistor at each bit in the column. When writing, the row lines contain the state the capacitor should take on. When reading, the sense-amplifier determines the level of charge in the capacitor. If it is more than 50 percent, it reads it as a 1; otherwise it reads it as a 0. The counter tracks the refresh sequence based on which rows have been accessed in what order. The length of time necessary to do all this is so short that it is expressed in nanoseconds (billionths of a second). A memory chip rating of 70ns means that it takes 70 nanoseconds to completely read and recharge each cell. Memory cells alone would be worthless without some way to get information in and out of them. So the memory cells have a whole support infrastructure of other specialized circuits. These circuits perform functions such as: Identifying each row and column (row address select and column address select) Keeping track of the refresh sequence (counter) Reading and restoring the signal from a cell (sense amplifier) Telling a cell whether it should take a charge or not (write enable) Other functions of the memory controller include a series of tasks that include identifying the type, speed and amount of memory and checking for errors. Static RAM works differently from DRAM. We'll look at how in the next section. http://computer.howstuffworks.com/ram1.htm Graphics Processing Units While we group graphics cards with the "guts" of a computer, some have their own microprocessors called a graphics processing unit (GPU). The GPU can handle some of the processing power required to generate rich graphics, which takes some of the load off the CPU.

The base of operations for the brains of a computer is the motherboard. The motherboard serves as a literal foundation for many of the other elements inside your computer. It's a large printed circuit board. The motherboard provides the connections and sockets that let other components communicate with each other. Motherboards come in different shapes and sizes -- a motherboard in a laptop computer might not look like one from a desktop PC.

The computer brain is a microprocessor called the central processing unit (CPU). The CPU is a chip containing millions of tiny transistors. It's the CPU's job to perform the calculations necessary to make the computer work -- the transistors in the CPU manipulate the data. You can think of a CPU as the decision maker.

This photo shows the RAM (upper left), the wireless card (bottom center) and the small battery (bottom right).

Another critical component in computers is memory. The two most important kinds of memory are read-only memory (ROM) and random access memory (RAM). Computers can read data stored in ROM, but can't write new data to it. With RAM, computers can read from and write to that memory. Without computer memory, every calculation on a computer would be stateless. That means there'd be no way to preserve information from one moment to the next and every process would start on a clean slate. That's not useful if you want to create complex programs.

Many desktop PCs have the capacity for additional RAM. The user simply has to open the computer and plug RAM chips into the appropriate sockets on the motherboard. But other computers are sealed systems -- you aren't meant to open them and make changes so you're pretty much stuck with what you've got.

A chip called the Basic Input/Output System (BIOS) works closely with the CPU. BIOS is a specific kind of ROM. If you think of the CPU as the brain of the computer, then you might consider BIOS to be the spine. It's the job of BIOS to handle interactions between the software running on a computer and the machine's hardware components.

The motherboard, CPU, ROM, RAM and BIOS handle most of the heavy lifting for computer processes. They're in charge of allocating resources to applications so that they run smoothly. They also accept input from devices like keyboards, mice and other computer accessories.

Go to the following site

http://computer.howstuffworks.com/hard-disk.htm

http://computer.howstuffworks.com/hard-disk.htm/printable

How the CD Rom Works

Look at the following sites

http://computer.howstuffworks.com/cd.htm http://computer.howstuffworks.com/cd.htm/printable

http://howthingswork.virginia.edu/

9. Label the following parts

Look at more computer components such as the Floppy Drive and label the parts

Explain the following in terms of a modern 9cm floppy disk and its drive.

Stepper Motor

Track

Sector

Block

10. Give the name for each of the following symbols in Logical Data Flow Diagrams

Understanding the CD: Material

As discussed in How Analog and Digital Recording Works, a CD can store up to 74 minutes of music, so the total amount of digital data that must be stored on a CD is:

44,100 samples/channel/second x 2 bytes/sample x 2 channels x 74 minutes x 60 seconds/minute = 783,216,000 bytes

To fit more than 783 megabytes (MB) onto a disc only 4.8 inches (12 cm) in diameter requires that the individual bytes be very small. By examining the physical construction of a CD, you can begin to understand just how small these bytes are.

A CD is a fairly simple piece of plastic, about four one-hundredths (4/100) of an inch (1.2 mm) thick. Most of a CD consists of an injection-molded piece of clear polycarbonate plastic. During manufacturing, this plastic is impressed with microscopic bumps arranged as a single, continuous, extremely long spiral track of data. We'll return to the bumps in a moment. Once the clear piece of polycarbonate is formed, a thin, reflective aluminum layer is sputtered onto the disc, covering the bumps. Then a thin acrylic layer is sprayed over the aluminum to protect it. The label is then printed onto the acrylic. A cross section of a complete CD (not to scale) looks like this:

Cross-section of a CD

Understanding the CD: The Spiral

A CD has a single spiral track of data, circling from the inside of the disc to the outside. The fact that the spiral track starts at the center means that the CD can be smaller than 4.8 inches (12 cm) if desired, and in fact there are now plastic baseball cards and business cards that you can put in a CD player. CD business cards hold about 2 MB of data before the size and shape of the card cuts off the spiral.

What the picture on the right does not even begin to impress upon you is how incredibly small the data track is -- it is approximately 0.5 microns wide, with 1.6 microns separating one track from the next. (A micron is a millionth of a meter.) And the bumps are even more miniscule...

Understanding the CD: Bumps

The elongated bumps that make up the track are each 0.5 microns wide, a minimum of 0.83 microns long and 125 nanometers high. (A nanometer is a billionth of a meter.) Looking through the polycarbonate layer at the bumps, they look something like this:

You will often read about "pits" on a CD instead of bumps. They appear as pits on the aluminum side, but on the side the laser reads from, they are bumps.

The incredibly small dimensions of the bumps make the spiral track on a CD extremely long. If you could lift the data track off a CD and stretch it out into a straight line, it would be 0.5 microns wide and almost 3.5 miles (5 km) long!

To read something this small you need an incredibly precise disc-reading mechanism. Let's take a look at that.

CD Player Components

The CD player has the job of finding and reading the data stored as bumps on the CD. Considering how small the bumps are, the CD player is an exceptionally precise piece of equipment. The drive consists of three fundamental components:

A drive motor spins the disc. This drive motor is precisely controlled to rotate between 200 and 500 rpm depending on which track is being read.

A laser and a lens system focus in on and read the bumps.

A tracking mechanism moves the laser assembly so that the laser's beam can follow the spiral track. The tracking system has to be able to move the laser at micron resolutions.

Inside a CD player

What the CD Player Does: Laser Focus

Inside the CD player, there is a good bit of computer technology involved in forming the data into understandable data blocks and sending them either to the DAC (in the case of an audio CD) or to the computer (in the case of a CD-ROM drive).

The fundamental job of the CD player is to focus the laser on the track of bumps. The laser beam passes through the polycarbonate layer, reflects off the aluminum layer and hits an opto-electronic device that detects changes in light. The bumps reflect light differently than the "lands" (the rest of the aluminum layer), and the opto-electronic sensor detects that change in reflectivity. The electronics in the drive interpret the changes in reflectivity in order to read the bits that make up the bytes.

What the CD Player Does: Tracking

The hardest part is keeping the laser beam centered on the data track. This centering is the job of the tracking system. The tracking system, as it plays the CD, has to continually move the laser outward. As the laser moves outward from the center of the disc, the bumps move past the laser faster -- this happens because the linear, or tangential, speed of the bumps is equal to the radius times the speed at which the disc is revolving (rpm). Therefore, as the laser moves outward, the spindle motor must slow the speed of the CD. That way, the bumps travel past the laser at a constant speed, and the data comes off the disc at a constant rate.

CD Encoding Issues

If you have a CD-R drive, and want to produce your own audio CDs or CD-ROMs, one of the great things you've got going in your favor is the fact that software can handle all the details for you. You can say to your software, "Please store these songs on this CD," or "Please store these data files on this CD-ROM," and the software will do the rest. Because of this, you don't need to know anything about CD data formatting to create your own CDs. However, CD data formatting is complex and interesting, so let's go into it anyway. Photo courtesy Ernest von Rosen, AMGmedia Recordable CD

To understand how data are stored on a CD, you need to understand all of the different conditions the designers of the data encoding methodology were trying to handle. Here is a fairly complete list:

Because the laser is tracking the spiral of data using the bumps, there cannot be extended gaps where there are no bumps in the data track. To solve this problem, data is encoded using EFM (eight-fourteen modulation). In EFM, 8-bit bytes are converted to 14 bits, and it is guaranteed by EFM that some of those bits will be 1s.

Because the laser wants to be able to move between songs, data needs to be encoded into the music telling the drive "where it is" on the disc. This problem is solved using what is known as subcode data. Subcode data can encode the absolute and relative position of the laser in the track, and can also encode such things as song titles.

Because the laser may misread a bump, there need to be error-correcting codes to handle single-bit errors. To solve this problem, extra data bits are added that allow the drive to detect single-bit errors and correct them.

Because a scratch or a speck on the CD might cause a whole packet of bytes to be misread (known as a burst error), the drive needs to be able to recover from such an event. This problem is solved by actually interleaving the data on the disc, so that it is stored non-sequentially around one of the disc's circuits. The drive actually reads data one revolution at a time, and un-interleaves the data in order to play it.

If a few bytes are misread in music, the worst thing that can happen is a little fuzz during playback. When data is stored on a CD, however, any data error is catastrophic. Therefore, additional error correction codes are used when storing data on a CD-ROM.

CD Data Formats

There are several different formats used to store data on a CD, some widely used and some long-forgotten. The two most common are CD-DA (audio) and CD-ROM (computer data).

Sound Basics

To understand how speakers work, you first need to understand how sound works.

Inside your ear is a very thin piece of skin called the eardrum. When your eardrum vibrates, your brain interprets the vibrations as sound -- that's how you hear. Rapid changes in air pressure are the most common thing to vibrate your eardrum.

An object produces sound when it vibrates in air (sound can also travel through liquids and solids, but air is the transmission medium when we listen to speakers). When something vibrates, it moves the air particles around it. Those air particles in turn move the air particles around them, carrying the pulse of the vibration through the air as a traveling disturbance.

To see how this works, let's look at a simple vibrating object -- a bell. When you ring a bell, the metal vibrates -- flexes in and out -- rapidly. When it flexes out on one side, it pushes out on the surrounding air particles on that side. These air particles then collide with the particles in front of them, which collide with the particles in front of them and so on. When the bell flexes away, it pulls in on these surrounding air particles, creating a drop in pressure that pulls in on more surrounding air particles, which creates another drop in pressure that pulls in particles that are even farther out and so on. This decreasing of pressure is called rarefaction.

In this way, a vibrating object sends a wave of pressure fluctuation through the atmosphere. When the fluctuation wave reaches your ear, it vibrates the eardrum back and forth. Our brain interprets this motion as sound.

Networks

Star Topology All devices are connected to a central hub. Nodes communicate across the network by passing data through the hub.

Bus Topology All devices are connected to a central cable, called the bus or backbone.

Ring Topology All devices are connected to one another in the shape of a closed loop, so that each device is connected directly to two other devices, one on either side of it.

11. Label the parts of the Hard Disk Drive below at the end of the five pointers.

How the CD Rom Works

Give the name for each of the following symbols in Logical Data Flow Diagrams

Answer is here

Bibliography

http://www.wisegeek.com/how-do-computers-work.htm

http://computer.howstuffworks.com/cd.htm http://computer.howstuffworks.com/cd.htm/printable

http://howthingswork.virginia.edu/

http://computer.howstuffworks.com/hard-disk.htm

http://computer.howstuffworks.com/hard-disk.htm/printable