SOLUTIONS

INPUT AND OUTPUT ANSWERS

1. Two categories of input devices are pointing devices and character devices. Give two examples of each type.

4 x 1 mark = 4 marks

(a). Pointing Devices (any two of the following):

- mouse

- trackball

- joystick

- pen input devices

- touch screen	2 x 1 mark = 2 marks

(b) Character Devices (any two of the following):

- bar code wand

- optical character reader

- document reader	2 x 1 mark = 2 marks

4 marks

2. Briefly explain the difference between how a laser printer and an ink-jet printer work.

2 x 2 marks = 4 marks

| Laser Printer | Ink-Jet |
| --- | --- |
| Data from the computer is converted into a laser beam that is directed by a mirror to a photosensitive drum. The areas on the drum touched by the laser attract toner particles that are transferred to the paper. The toner is fused to the paper with heat and pressure. | Sprays tiny drops of ink onto the paper. The print head contains a nozzle with between 50 and 100 small holes. |

3. With the development of multimedia, special devices are required to capture image and sound data. One of these is a digital camera and another is voice recognition software. Describe what each of these devices do and give an example of where each has an application.

2 x 2 marks = 4 marks

(a) Digital Camera:

Records photographs digitally where no film is used. The photographs can be copied directly to a computer and stored.	1 mark

Uses:	desktop publishing, recording documents, databases, id cards, presentations

1 mark

(b). Voice Recognition:

Speak through microphone and analogue to digital conversion, and digitised voice is compared against patterns stored on computer. Unrecognised words are presented to the user to identify.	1 mark

# Uses:	Computer security, computer commands, speech to text, data entry	1 mark

4 x 1 mark = 4 marks

4. The production of special hardware devices for disabled individuals has greatly improved their capabilities with computer technology. Describe two alternative input devices that people with motor disabilities can use instead of the standard keyboard.

2 x 2 marks = 4 marks

Individuals who have use of one finger, or have access to a mouth- or head-stick or some other pointing device, can control the computer by pressing keys with the pointing device for computer commands,

Mounting keyboards perpendicular to tables or wheelchair trays at head-height can assist individuals with limited mobility

Keyboard specific left- and right-handed keyboards with expanded keyboards (larger keys spaced far apart) can replace standard keyboards for those with limited fine motor control.

Braille input devices are available. Braille key labels can assist with keyboard use or speech to text, data entry.

Any two 2 x 2 marks = 4 marks

5. Explain each of the following:

- OMR

- OCR

- MICR

- LCD

- SVGA

5 x 1 mark = 5 marks

- (a)	OMR:	Optical Mark Reader

- (b)	OCR:  	Optical Character Recognition

- (c)	MICR: 	Magnetic Ink Character Recognition

- (d)	LCD:		Liquid Crystal Display

- (e)	SVGA:	Super Video Graphics Array

- 5 x 1 mark = 5 marks

6. When purchasing a computer system, much thought also needs to be given to choice of printer. Describe four factors, other than price, that should be taken into account when choosing a printer.

4 x 1 mark = 4 marks

6.  Any four of the following:

- Speed - number of pages per minute

- Resolution - dots per inch

- What type of output - paper, transparencies, labels, graphics

- Colour

- Price of consumables - ribbons, toner, ink refills

- Noise

- Warranty

- Reputation of supplier/manufacturer

4 x 1 mark = 4 marks

4 + 4 + 4 + 4 + 5 + 4 = Total: 25 marks

ANSWERS

Character Devices (any two of the following):

- bar code wand

- optical character reader

- document reader	2 x 1 mark = 2 marks

4 marks

2.

| Laser Printer | Ink-Jet |
| --- | --- |
| Data from the computer is converted into a laser beam that is directed by a mirror to a photosensitive drum. The areas on the drum touched by the laser attract toner particles that are transferred to the paper. The toner is fused to the paper with heat and pressure. | Sprays tiny drops of ink onto the paper. The print head contains a nozzle with between 50 and 100 small holes. |

2 x 2 marks = 4 marks

3. Digital Camera:

Records photographs digitally where no film is used. The photographs can be copied directly to a computer and stored.	1 mark

Uses:

desktop publishing, recording documents, databases, id cards, presentations

1 mark

4. Voice Recognition:

Speak through microphone and analog to digital conversion, and digitised voice is compared against patterns stored on computer. Unrecognised words are presented to the user to identify.	1 mark

# Uses:

security, computer commands, speech to text, data entry	1 mark

4 x 1 mark = 4 marks

5.

- OMR:	Optical Mark Reader

(b)	OCR:  	Optical Character Recognition

(c)	MICR: 	Magnetic Ink Character Recognition

(d)	LCD:	Liquid Crystal Display

(e)	SVGA:	Super Video Graphics Array

5 x 1 mark = 5 marks

6.  Any four of the following:

- Speed - number of pages per minute

- Resolution - dots per inch

- What type of output - paper, transparencies, labels, graphics

- Colour

- Price of consumables - ribbons, toner, ink refills

- Noise

- Warranty

- Reputation of supplier/manufacturer

4 x 1 mark = 4 marks

4 + 4 + 4 + 4 + 5 + 4 = Total: 25 marks